| Name | Year | Purpose of the Study | Methodology | Limitations | Relevance to Our Work |
| --- | --- | --- | --- | --- | --- |
| Adaptive Traffic Signaling Control Using SUMO Simulator | 2025 | To develop a two-stage, three-module fuzzy logic system for real-time management of signalized junctions to reduce average vehicle waiting times. | Implemented a fuzzy logic traffic management system using the SUMO simulation tool with real-world traffic data from a congested intersection in Kilis, Turkey. The system dynamically adjusts signal timing using 52 fuzzy rules and was compared against conventional fixed-time control. | The study concentrates exclusively on an isolated intersection rather than a multi-intersection network. | Proves dynamic signal control in SUMO outperforms fixed-time systems. Uses real-time vehicle counts and queue lengths to adjust timings. |
| A SUMO Based Simulation Framework for Intelligent Traffic Management System | 2020 | To explore a Deep-Neuro-Fuzzy model within the SUMO traffic simulation environment and develop a GUI to control simulation input attributes for optimal route planning. | Utilized C-means clustering and a Deep-Neuro-Fuzzy framework to generate dynamic road weights based on environmental and artificial attributes. Applied routing algorithms (Dijkstra, A*, CHWrapper) in SUMO to find optimal vehicle paths. | The model has not yet been implemented in real-time physical road networks, which would require costly personalized infrastructure. Other hidden parameters of existing SUMO algorithms remained unchanged during testing. | Shows how to integrate SUMO with an external backend to update traffic logic. Validates extracting live metrics and sending custom commands back to SUMO |
| AI-Powered Traffic Signal Control: A Deep Reinforcement Learning Framework for Urban Congestion Reduction | 2025 | To explore AI-based traffic signal regulation using deep reinforcement learning to minimize vehicle waiting times, maximize flow efficiency, and enhance pedestrian and emergency vehicle safety.. | Formulated traffic signal optimization as a sequential decision-making task using DRL techniques like Proximal Policy Optimization (PPO), Deep Q-Networks (DQN), and Advantage Actor-Critic (A3C). Performed simulations using SUMO with real-world and synthetic data to compare AI methods against fixed-timing controls. | Relied on simulation-based testing, which did not fully account for erratic driving habits, infrastructural limitations, or external weather factors | Highlights using SUMO to prioritize emergency vehicles and specific vehicle classes. Confirms the method of using live queue data to optimize traffic lights. |

| Name | Year | Purpose of the Study | Methodology | Limitations | Relevance to Our Work |
| --- | --- | --- | --- | --- | --- |
| Enhancing Traffic Monitoring Systems through Image Analysis and Rule-Based Machine Learning | 2025 | To present an advanced traffic monitoring system integrating reasoning-based image analysis and rule-based machine learning to improve traffic management, reduce congestion, and enhance road safety. | Leverages visual data from spot sensors utilizing a three-tier module approach. It processes daytime scenes via Google cloud using the RCNN algorithm and nighttime scenes via bimodal histograms and Otsu's thresholding for headlight extraction. A high-level module applies rule-based reasoning (via an inference engine and knowledge base) to refine the models. | Real-world testing was restricted to favorable weather, meaning the system's robustness in environmental conditions like fog or snow remains untested. | It proves that adding rule-based reasoning helps systems instantly adapt to unpredictable, real-time traffic scenarios. |